8-Bit Digital Calculator Using Logisim Evolution

1. Introduction

A calculator is a device that takes numbers from the user, performs an arithmetic operation on them, and shows the answer. In a digital calculator, this whole job is done by digital circuits instead of by a person. There is no software inside our project. Every step — taking the input, storing it, adding or subtracting it, and showing the answer — is done by logic components connected together.

Digital circuits work with only two values, 0 and 1. A 0 means a low voltage and a 1 means a high voltage. Numbers, control signals and even the pattern shown on a display are all built from these two values. Because a circuit can only understand 0 and 1, a decimal number typed by the user must first be changed into binary form, and the binary answer must be changed back into decimal form before it can be displayed.

This project presents an 8-bit digital calculator that was designed and simulated in Logisim Evolution. The user enters numbers using input buttons, selects addition or subtraction, and presses the result button. The circuit stores the numbers, performs the operation, converts the answer into a display format, and shows it on 7-segment displays. Clear and back buttons are also provided so that the user can correct a mistake while typing.

2. Objectives

- To design an 8-bit digital calculator using digital logic components.

- To understand how binary numbers are processed inside a digital circuit.

- To use logic gates and other components to perform calculations.

- To understand the working of registers, adders, subtractors and multiplexers.

- To display the final result using 7-segment displays.

- To simulate and test the complete circuit in Logisim Evolution.

3. Basic Theory

3.1 Digital Logic

Digital logic works with only two values, 0 and 1. Logic gates take these values as input and produce an output according to a fixed rule. By joining many gates together, we can build larger circuits that can store data, compare data or perform arithmetic. Our calculator is built in exactly this way, from small logic blocks joined into bigger ones.

3.2 Binary Number

A binary number uses only the digits 0 and 1. Digital circuits use binary because an electronic signal can easily stay in two states, LOW and HIGH. It is much simpler and much more reliable to detect two states than to detect ten different levels for the ten decimal digits.

3.3 8-Bit System

An 8-bit system uses eight binary digits together as one value. Eight bits give 256 different combinations, so an 8-bit value can represent numbers from 0 to 255. Our calculator must also be able to show a negative answer, for example when a smaller number is subtracted from a larger one. For this reason the eight bits are used in signed form, where the leftmost bit shows the sign. In signed form the range becomes −128 to +127.

3.4 Logic Gates

The three basic gates are AND, OR and NOT. An AND gate gives 1 only when all of its inputs are 1. An OR gate gives 1 when at least one of its inputs is 1. A NOT gate simply reverses its input, changing 0 into 1 and 1 into 0. In this project, gates are used to control which button is active, to decide when a value should be stored, and to build the conversion blocks used by the display.

4. Project Components

The table below lists the main components used in the calculator and explains the job of each one.

| Component | Purpose in the project |
| --- | --- |
| Number buttons (0–9) | Ten buttons used to type a number. Each button sends its own digit value into the circuit. |
| Add and Subtract buttons | Used to select the operation. The choice is stored in the circuit until the result button is pressed. |
| Clear and Back buttons | Clear removes the numbers and starts again. Back removes the last digit that was typed. |
| Logic gates | AND, OR and NOT gates control the buttons, decide when to store a value, and build the conversion blocks. |
| Registers | Two 8-bit registers. One holds the number being typed, and the other holds the first number of the calculation. |
| Adder | Adds the two stored numbers and gives the sum. |
| Subtractor | Subtracts the second number from the first and gives the difference. |
| Multiplexer | Acts like a switch. It chooses which value should pass forward, for example the sum or the difference. |
| Binary to BCD section | Changes the binary answer into separate decimal digits so that it can be displayed. |
| Decoder | Takes one decimal digit and turns on the correct segments needed to draw that digit. |
| 7-segment displays | Show the final answer to the user. One display is used for the minus sign. |

Table 1: Main components used in the 8-Bit Digital Calculator.

5. Working Principle

The complete calculator works in the following steps.

- The user enters the first number using the input buttons.

- The entered value is stored inside the circuit in a register.

- The user selects Addition or Subtraction. The first number moves to the second register and the operation is stored.

- The user enters the second number, and it is stored in the same way as the first.

- When the result button is pressed, the arithmetic section performs the selected operation.

- The answer is converted into a suitable display format by the BCD conversion section.

- The decoder controls the 7-segment displays and lights the correct segments.

- The final answer is shown to the user, with the minus sign lit if the answer is negative.

The overall flow of data through the circuit is shown below.

INPUT

↓

CONTROL

↓

REGISTERS

↓

ADDER / SUBTRACTOR

↓

BCD CONVERSION

↓

DECODER

↓

7-SEGMENT DISPLAY

6. Logisim Implementation

The complete circuit was designed and simulated in Logisim Evolution. Logisim is a simulation tool that lets us place digital components on a workspace, connect them with wires, change the inputs by clicking, and immediately see the outputs. Because everything runs in simulation, the design can be built and corrected many times without using any physical hardware or spending money on parts.

The design was built in parts. Small blocks such as the conversion block and the decoder block were made first and tested on their own. After each block was found to be working, all of them were joined together to make the complete calculator circuit.

The main sections that can be seen in the circuit are described below.

- Input section: the group of buttons used to type the digits and to select the operation. Each button sends its own signal into the circuit.

- Control section: the flip-flops and gates that decide when a value should be stored and which operation has been selected. It makes sure that one press stores one digit only.

- Register section: the two 8-bit registers that hold the numbers used in the calculation.

- Arithmetic section: the adder, the subtractor and the multiplexer that produce and select the answer.

- Display section: the BCD conversion block, the decoders and the 7-segment displays that show the answer in decimal form.

7. Result

The circuit was tested by giving different inputs and selecting the required operation each time. For example, entering 47, pressing Add, entering 25 and pressing the result button showed 72 on the display. Entering 12, pressing Subtract, entering 30 and pressing the result button showed 18 with the minus sign lit, which is the correct answer. The clear and back buttons were also tested and worked as expected.

In every test, the answer processed by the circuit appeared correctly on the 7-segment displays. The simulation shows that different digital logic components — gates, registers, adders, subtractors, multiplexers, converters and decoders — can work together to perform a useful calculation.

| Input given | Operation | Displayed result |
| --- | --- | --- |
| 47 and 25 | Addition | 72 |
| 9 and 4 | Subtraction | 5 |
| 12 and 30 | Subtraction | −18 |
| 100 and 27 | Addition | 127 |

Table 2: Sample test results of the calculator.

8. Conclusion

The project successfully demonstrates the design of an 8-bit digital calculator using digital logic. The circuit accepts numbers from the user, performs addition and subtraction, and displays the answer correctly on 7-segment displays.

While completing this project, we understood binary numbers, logic gates, registers, adders and subtractors, multiplexers, BCD and display conversion, 7-segment displays, and the use of a simulation tool for building digital circuits. Most importantly, we saw how these separate topics fit together in one working design.

This project gives a basic understanding of how digital electronic systems perform calculations.

9. Future Improvement

- More arithmetic operations, such as multiplication and division, can be added to the calculator.

- A better keypad and input system can be used to make typing numbers easier.

- The circuit can be implemented using real hardware, so that it works outside the simulator.